SMART RAIL TICKETING SYSTEM USING RFID

G. NANDHAGOPAL (23322001)

M. MADHAVAN (23322008)

S. RAVIN RAJ (23322009)

# INTERNET OF THINGS AND ROBOTICS LAB

COURSE TEACHER: Dr.T.Kalaiselvi, MCA.,Ph.D.,

## DEPARTMENT OF COMPUTER SCIENCE AND APPLICATIONS

THE GANDHIGRAM RURAL INSTITUTE

(DEEMED TO BE UNIVERSITY)

GANDHIGRAM-624302

# DECEMBER 2024

INDEX

| S.NO | CONTENTS | PAGE NO |
| --- | --- | --- |
| 1. | INTRODUCTION | 1 |
| 2. | COMPONENT LIST AND DESCRIPTION | 2 |
| 3. | CIRCUIT DIAGRAM | 9 |
| 4. | CODE AND EXPLANATION | 10 |
| 5. | WORKING PRINCIPLE | 19 |
| 6. | ADVANTAGES | 20 |
| 7. | CONCLUSION AND FUTURE WORK | 21 |
| 8. | SCREENSHOT | 22 |
| 9. | REFERENCE | 25 |

- INTRODUCTION :

This project utilizes Radio Frequency Identification (RFID) technology to streamline ticketing processes, improve operational efficiency, and enhance passenger convenience. The system is built using a NodeMCU microcontroller, which connects to a real-time database via Firebase for secure and efficient data handling. The system operates by integrating an RFID reader to identify passengers through unique RFID tags. Upon identification, the NodeMCU communicates with Firebase to verify ticket details, such as validity and active status. Once authenticated, a servo motor is triggered to allow physical access to the platform or train .

- PROJECT DESCRIPTION :

2.1. COMPONENTS USED :

    - NodeMCU ESP8266

    - MFRC522

    - Servo Motor

    - Jumper Wires

NODEMCU ESP8266 :

The ESP8266 NodeMCU is a powerful and affordable IoT development board that integrates the ESP8266 Wi-Fi module with a microcontroller, making it a versatile platform for building connected devices. It features built-in Wi-Fi, allowing it to connect directly to the internet or serve as an access point for local networks. The board is equipped with multiple GPIO pins, enabling easy integration with sensors, actuators, and other peripherals. Programmable via the Arduino IDE or Lua scripting language, the NodeMCU supports various communication protocols such as MQTT, HTTP, and WebSocket, making it ideal for IoT applications. Its compact, breadboard-friendly design and support for a wide range of libraries and tools make it beginner-friendly yet powerful enough for advanced projects. With features like low power consumption, 4MB of flash memory, and compatibility with cloud platforms, the NodeMCU is widely used in smart home automation, environmental monitoring, and wearable technology projects. Its affordability and versatility make it a popular choice for hobbyists and developers alike.

MFRC522 :

The MFRC522 is a popular RFID module based on the NXP MFRC522 IC, designed for short-range communication using RFID technology. It operates on a 13.56 MHz frequency, enabling it to read and write data to RFID tags and cards that comply with the ISO/IEC 14443 standard. Compact and affordable, the module is widely used in access control systems, attendance management, payment systems, and other contactless applications. The MFRC522 communicates with microcontrollers via SPI, I2C, or UART interfaces, making it highly versatile and compatible with platforms like Arduino, Raspberry Pi, and NodeMCU. With low power consumption and a typical range of 2-5 cm, it’s ideal for projects requiring secure, short-range data exchange.

The module includes built-in anti-collision protocols to handle multiple RFID tags within its range efficiently. It also supports several encryption methods, such as MIFARE Classic, to enhance data security. The MFRC522 is easy to integrate, requiring minimal wiring and setup, and comes with libraries for most microcontroller platforms to simplify programming. Its compact design and reliable performance make it an excellent choice for prototyping or deploying cost-effective RFID-based systems in both hobbyist and professional projects.

SERVO MOTOR :

A servo motor is a rotary or linear actuator that allows precise control of angular or linear position, speed, and acceleration. It consists of a motor coupled with a sensor for position feedback, a control circuit, and sometimes gears for increased torque. Servo motors are commonly used in robotics, automation, remote-controlled devices, and industrial machinery due to their ability to achieve accurate and repeatable movements. They operate by receiving a Pulse Width Modulation (PWM) signal, which determines the position of the motor shaft. For example, in standard hobby servo motors, a 1-2 ms pulse corresponds to a rotation angle between 0° and 180°, though some servos can rotate 360° or be modified for continuous rotation.

Servo motors come in various types, such as positional rotation, continuous rotation, and linear servos, to suit different applications. Positional rotation servos are typically used in robotic arms or steering mechanisms, while continuous rotation servos are suitable for wheeled robots. Linear servos are often found in actuators requiring straight-line motion. Their compact size, low power consumption, and versatility make them indispensable in a wide range of projects. The ability to maintain precise positioning and react quickly to control signals makes servo motors an essential component in systems requiring controlled motion.

JUMPER WIRES :

Jumper wires are essential components used in electronics and prototyping to create temporary or permanent connections between points in a circuit. They are flexible insulated wires with solid metal tips, which can be connected to breadboards, headers, or other components without the need for soldering. Jumper wires come in three types based on their end connectors: male-to-male, female-to-female, and male-to-female, allowing them to adapt to different pin configurations and devices.

These wires are available in various lengths and colors, helping users organize and troubleshoot circuits more efficiently. They are particularly useful for prototyping with development boards like Arduino, Raspberry Pi, or NodeMCU, enabling quick experimentation and modification of circuit designs. Durable and reusable, jumper wires are a staple in electronics kits for beginners and professionals alike, offering a simple and reliable way to interconnect components in projects.

2.2. SOFTWARE DESCRIPTION :

    - Arduino IDE

    - Firebase

    - Twilio

ARDUINO IDE:

The Arduino Integrated Development Environment (IDE) is a user-friendly platform designed for programming Arduino boards and compatible microcontrollers. It provides a simple interface where users can write, edit, and upload code (sketches) to their Arduino devices using a USB connection. The IDE supports the Arduino programming language, which is a simplified version of C/C++, making it accessible for beginners while still powerful enough for advanced users. Key features include syntax highlighting, code suggestions, a serial monitor for debugging, and a vast library manager that allows easy integration of third-party libraries and hardware.

Available for Windows, macOS, and Linux, the Arduino IDE is open-source, ensuring continuous improvement and community support. Its versatility extends beyond Arduino boards, as it can be configured to work with other microcontroller platforms like ESP8266 and ESP32. The IDE’s built-in examples and extensive online documentation make it an excellent starting point for those new to embedded programming. With its robust ecosystem of libraries, tools, and community-driven projects, the Arduino IDE has become a cornerstone of innovation in DIY electronics and professional prototyping.

FIREBASE :

Firebase is a comprehensive backend-as-a-service (BaaS) platform developed by Google, designed to simplify app development with its suite of tools and services. It provides a wide range of features, including real-time databases, cloud storage, user authentication, and analytics, enabling developers to build scalable, feature-rich applications without managing complex backend infrastructure. Firebase's Realtime Database and Cloud Firestore allow seamless synchronization of data across devices in real-time, making it ideal for chat apps, collaborative tools, and live-updating dashboards. Additionally, Firebase Authentication simplifies user login through email, phone, or social media platforms, offering robust security and easy integration.

Beyond its core services, Firebase also supports performance monitoring, crash reporting, and cloud messaging for push notifications, enhancing the app development and user experience. With built-in machine learning features through Firebase ML, developers can easily incorporate AI-driven functionalities like image recognition or language translation. Firebase’s compatibility with iOS, Android, and web platforms makes it a versatile solution for cross-platform development. Its serverless architecture, combined with powerful APIs, enables developers to focus on creating engaging apps while Firebase handles the backend complexities. This makes Firebase a preferred choice for both startups and established companies aiming to build and scale their applications efficiently.

TWILIO :

Twilio is a cloud-based communications platform that enables developers to integrate messaging, voice, and video functionalities into their applications through simple APIs. With Twilio, businesses can send SMS, make and receive phone calls, manage chat messages, and even implement two-factor authentication (2FA) effortlessly. Its flexibility allows developers to create highly customized communication workflows tailored to specific use cases, such as customer support, appointment reminders, or marketing campaigns. Twilio’s global infrastructure ensures reliable and scalable communication across over 100 countries, making it a preferred choice for both small businesses and large enterprises.

In addition to its core messaging and voice services, Twilio offers advanced tools like Twilio Studio for building interactive workflows without writing extensive code, Twilio Flex for customizable contact center solutions, and Twilio Verify for secure user authentication. Twilio also provides detailed analytics and reporting to help businesses optimize their communication strategies. Its pay-as-you-go pricing model eliminates the need for upfront investments, making it accessible for developers and businesses of all sizes. With robust documentation, strong community support, and compatibility with multiple programming languages, Twilio empowers developers to create seamless and innovative communication experiences for users worldwide.

3. CIRCUIT DIAGRAM :

4.CODE AND EXPLANATION :

// Header Files

#include <SPI.h>

#include <MFRC522.h>

#include <ESP8266WiFi.h>

#include <Firebase_ESP_Client.h>

#include <String.h>

#include <ESP8266HTTPClient.h>

#include <WiFiClientSecureBearSSL.h>

#include <typeinfo>

#include <Servo.h>

Servo servo;    //Servo motor object

// Twilio credentials

const String account_sid = "AC4e615088cdca4c3f009013514bce205b";

const String auth_token = "e8cceed744cf6ad128757087547bbe68";

const String from_number = "+17813556016";  // Your Twilio phone number

String to_number = "";    // The recipient phone number

// RFID Pin Configuration

#define SS_PIN D4

#define RST_PIN D3

MFRC522 rfid(SS_PIN, RST_PIN);

MFRC522::MIFARE_Key key;

// Variables to store name, mobile and ticketid

char name[17];

char tkid[17];

char mob[17];

// Wifi Credentials

const char* WIFI_SSID = "Rohit Maddy";

const char* WIFI_PASS = "12345678";

// Firebase Credentials

#define API_KEY "AIzaSyB3eI40212LAl18wWFgSsDEMRsXdIBwXHg"

#define DB_URL "rfidtrainticket-e67f9-default-rtdb.firebaseio.com/"

// Firebase Authentication Credentials

#define USER_EMAIL "chatwithravin@gmail.com"

#define PW "Ravin170403$"

// Firebase Objects

FirebaseData fbdo;

FirebaseConfig config;

FirebaseAuth auth;

// Declare sendDataPrevMillis globally

unsigned long sendDataPrevMillis = 0;

void setup() {

Serial.begin(9600); // Baudrate Declaration

servo.attach(4);  // Servo motor Attach

SPI.begin();          // Init SPI bus

rfid.PCD_Init();      // Init MFRC522

Serial.println("Place your tag to the reader...");

// Initialize the default key (for most Mifare RFID tags, it's 0xFFFFFFFFFFFF)

for (byte i = 0; i < 6; i++) {

key.keyByte[i] = 0xFF;

}

WiFi.begin(WIFI_SSID, WIFI_PASS);             // Begin WiFi connection using SSID and password

while(WiFi.status() != WL_CONNECTED)    // Check if the WiFi status is not connected

{

delay(1000);                                // Wait 1 second between WiFi connection checks

Serial.println("Connecting to WiFi...");    // Print message indicating an attempt to connect to WiFi

}

Serial.println("Connected to WiFi.");         // Print message when WiFi connection is successful

Serial.print("IP Address: ");                 // Print the label for the IP address

Serial.println(WiFi.localIP());               // Print the assigned IP address

config.api_key = API_KEY;

// Assign the user sign-in credentials

auth.user.email = USER_EMAIL;

auth.user.password = PW;

// Assign the RTDB URL (required)

config.database_url = DB_URL;

fbdo.setBSSLBufferSize(4096, 4096);  // Provide both rx and tx buffer sizes

fbdo.setResponseSize(2048);

Firebase.begin(&config, &auth);

Firebase.setDoubleDigits(5);

config.timeout.serverResponse = 10 * 1000;

// Enable automatic network reconnection

Firebase.reconnectNetwork(true);

}

// Method to send SMS through Twilio

void sendSMS(String messageBody) {

if(WiFi.status() == WL_CONNECTED) {

std::unique_ptr<BearSSL::WiFiClientSecure>client(new BearSSL::WiFiClientSecure);

client->setInsecure();  // Ignore certificate verification (for simplicity)

HTTPClient https;

// Form the URL for Twilio API

String url = "https://api.twilio.com/2010-04-01/Accounts/" + account_sid + "/Messages.json";

// Begin HTTPS request

https.begin(*client, url);

// Set authorization headers (basic authentication)

https.setAuthorization(account_sid.c_str(), auth_token.c_str());

// Set content-type to x-www-form-urlencoded

https.addHeader("Content-Type", "application/x-www-form-urlencoded");

// Create the body for POST request

String body = "To=" + to_number + "&From=" + from_number + "&Body=" + messageBody;

// Send POST request

int httpResponseCode = https.POST(body);

// Handle the response

if (httpResponseCode > 0) {

String response = https.getString();

Serial.println("SMS sent successfully!");

Serial.println("Response: " + response);

} else {

Serial.println("Error sending SMS: " + String(httpResponseCode));

}

// Close connection

https.end();

}

else {

Serial.println("WiFi not connected");

}

}

void loop()

{

// Look for new card

if (!rfid.PICC_IsNewCardPresent()) {

return;

}

// Select the card

if (!rfid.PICC_ReadCardSerial()) {

return;

}

Serial.println("Card detected, reading data...");

byte len = 18;    // Buffer length (16 bytes data + 2 bytes CRC)

byte buffer[18];  // Buffer to store the data

// Authenticate and read data from block 4 (Name)

byte block = 4;  // Block to read (e.g., block 4 for name)

MFRC522::StatusCode status = rfid.PCD_Authenticate(MFRC522::PICC_CMD_MF_AUTH_KEY_A, block, &key, &(rfid.uid));

if (status != MFRC522::STATUS_OK) {

Serial.print(F("Authentication for block 4 failed: "));

Serial.println(rfid.GetStatusCodeName(status));

return;

}

// Read from block 4 (Name)

status = rfid.MIFARE_Read(block, buffer, &len);

if (status != MFRC522::STATUS_OK) {

Serial.print(F("Read from block 4 failed: "));

Serial.println(rfid.GetStatusCodeName(status));

return;

}

// Store the first part (Name) into the `name` variable

for (byte i = 0; i < 16; i++) {

name[i] = buffer[i];  // Copy the data from buffer to name variable

}

name[16] = '\0';  // Null-terminate the string

// Re-authenticate before reading from block 5 (ID)

block = 5;

status = rfid.PCD_Authenticate(MFRC522::PICC_CMD_MF_AUTH_KEY_A, block, &key, &(rfid.uid));

if (status != MFRC522::STATUS_OK) {

Serial.print(F("Authentication for block 5 failed: "));

Serial.println(rfid.GetStatusCodeName(status));

return;

}

// Read data from block 5 (ID)

status = rfid.MIFARE_Read(block, buffer, &len);

if (status != MFRC522::STATUS_OK) {

Serial.print(F("Read from block 5 failed: "));

Serial.println(rfid.GetStatusCodeName(status));

return;

}

// Store the ID into the `tkid` variable

for (byte i = 0; i < 16; i++) {

tkid[i] = buffer[i];  // Copy the data from buffer to id variable

}

tkid[16] = '\0';  // Null-terminate the string

block = 6;

status = rfid.PCD_Authenticate(MFRC522::PICC_CMD_MF_AUTH_KEY_A, block, &key, &(rfid.uid));

if (status != MFRC522::STATUS_OK) {

Serial.print(F("Authentication for block 5 failed: "));

Serial.println(rfid.GetStatusCodeName(status));

return;

}

// Read data from block 5 (ID)

status = rfid.MIFARE_Read(block, buffer, &len);

if (status != MFRC522::STATUS_OK) {

Serial.print(F("Read from block 5 failed: "));

Serial.println(rfid.GetStatusCodeName(status));

return;

}

for (byte i = 0; i < 16; i++) {

mob[i] = buffer[i];  // Copy the data from buffer to id variable

}

mob[16] = '\0';  // Null-terminate the string

block = 6;

status = rfid.PCD_Authenticate(MFRC522::PICC_CMD_MF_AUTH_KEY_A, block, &key, &(rfid.uid));

if (status != MFRC522::STATUS_OK) {

Serial.print(F("Authentication for block 6 failed: "));

Serial.println(rfid.GetStatusCodeName(status));

return;

}

// Read data from block 5 (ID)

status = rfid.MIFARE_Read(block, buffer, &len);

if (status != MFRC522::STATUS_OK) {

Serial.print(F("Read from block 5 failed: "));

Serial.println(rfid.GetStatusCodeName(status));

return;

}

// Print the values of name and ID

Serial.print("Name: ");

Serial.println(name);  // Print the name

Serial.print("Ticket ID: ");

Serial.println(tkid);    // Print the ID

Serial.print("Mobile:");

Serial.println(mob);

// Halt PICC (stop reading card)

rfid.PICC_HaltA();

rfid.PCD_StopCrypto1();

String moile = String(mob);

to_number=moile;

sendSMS("Welcome to RFID Railway Ticket. .Your ticket is Active. Happy Journey :)");

for (int i=0;i<=180;i++)

{

servo.write(i);

delay(10);

}

delay(3000);

for (int i=180;i>=0;i--)

{

servo.write(i);

delay(10);

}

if(Firebase.ready() && (millis() - sendDataPrevMillis > 1000 || sendDataPrevMillis == 0))

{

sendDataPrevMillis = millis();

// Sending data to Firebase

Firebase.RTDB.setString(&fbdo, "Mobile", mob);

Firebase.RTDB.setString(&fbdo, "Name", name);

Firebase.RTDB.setString(&fbdo, "TicketID", tkid);

Firebase.RTDB.setString(&fbdo, "Tstatus","ACTIVE");

}

}

5. WORKING PRINCIPLE :

When the passenger enters into the railway junction then they collect the ticket for their journey. Our innovation makes that ticket with RFID tag embedded one . By using this ticket the passenger is addressed to scan their RFID tickets on the particular scanner to get the validation on their tickets. When the scanner validation is successful then the platform gate will get opened for the entry of the valid passenger and send the SMS through the Twilio server to the passenger’s mobile number stated that “Your journey is started ”. When the passenger complete their journey , they are instructed to scan their RFID embedded ticket on the destination station. When the passenger’s ticket is valid then they get another SMS stated that their journey is completed .

6. ADVANTAGES :

An RFID-based railway ticketing system brings remarkable improvements in speed, efficiency, and user convenience. Its contactless nature allows passengers to have their tickets validated within seconds, significantly reducing queues and wait times during boarding. With RFID-enabled cards or devices, travellers can bypass the need for physical tickets, making the process smoother and more hassle-free. This automation not only saves time but also enhances the overall efficiency of railway operations by streamlining entry and exit procedures.

In addition to efficiency, the system offers enhanced security and accuracy. RFID tags are much harder to counterfeit compared to traditional paper tickets or QR codes, ensuring greater reliability in fare collection. Automated systems reduce the chances of human error in ticket verification and fare calculations. Furthermore, integration with digital payment systems enables seamless and automated transactions, making the ticketing process convenient for passengers and reducing operational costs for railway authorities. These advantages make RFID-based ticketing an innovative and forward-looking solution for modern railway systems.

7. CONCLUSION AND FUTUREWORK :

CONCLUSION

The implementation of an RFID-based railway ticketing system marks a significant step toward modernizing public transportation. By leveraging contactless technology, it enhances efficiency, convenience, and security in ticketing processes, benefiting both passengers and operators. The system reduces wait times, simplifies transactions, and minimizes errors, ensuring a seamless travel experience. Additionally, the improved resistance to counterfeiting and automation of payment processes make it a robust and reliable solution for railway networks.

FUTURE WORK

Future enhancements to the RFID-based railway ticketing system could focus on integrating it with mobile applications for real-time tracking and account management. The use of blockchain technology could further enhance transaction security and transparency. Additionally, expanding the system to support multi-modal transportation networks, such as buses and metros, can provide a unified and interconnected travel experience. Research into advanced RFID technology, such as long-range tags and enhanced environmental resistance, could also increase the system's adaptability and efficiency in diverse operational scenarios. We can generate fine amount for the passengers with their expired ticket through tracking them by measuring the distance of the locations.

8. SCREENSHOTS :

9.  REFERENCE :

https://www.twilio.com/en-us/blog/how-to-send-sms-messages-esp8266-cpp

https://www.javatpoint.com/iot-project-google-firebase-nodemcu

https://www.electronicwings.com/nodemcu/rfid-rc522-interfacing-with-nodemcu

https://www.instructables.com/Interfacing-Servo-Motor-With-NodeMCU/

https://forum.arduino.cc/t/esp8266-nodemcu-communication-failure-with-mfrc522-rfid-module/1183368